Homework26 (Network)

- 200.10.20.0/24

a. Branch_B - 60 hosts

Host bits: 6

CIDR: /26

Subnet mask: 255.255.255.192

Network address: 200.10.20.0

Broadcast address: 200.10.20.63
First Usable address: 200.10.20.1
Last Usable address: 200.10.20.62

b.Branch_C - 30 hosts

Host bits: 5

CIDR: /27

Subnet mask: 255.255.255.224

Network address: 200.10.20.64

Broadcast address: 200.10.20.95
First Usable address: 200.10.20.65
Last Usable address: 200.10.20.94

c.Branch_D - 12 hosts
Host bits: 4

CIDR: /28

Subnet mask: 255.255.255.240

Network address: 200.10.20.96

Broadcast address: 200.10.20.111
First Usable address: 200.10.20.97
Last Usable address: 200.10.20.110

d.Branch_A - 10 hosts
Host bits: 4

CIDR: /28

Subnet mask: 255.255.255.240

Network address: 200.10.20.112

Broadcast address: 200.10.20.127
First Usable address: 200.10.20.113
Last Usable address: 200.10.20.126

e. WAN_1 - 2 hosts
Host bits: 2

CIDR: /30

Subnet mask: 255.255.255.252

Network address: 200.10.20.128

Broadcast address: 200.10.20.131
First Usable address: 200.10.20.129
Last Usable address: 200.10.20.130

f. WAN_2 - 2 hosts
Host bits: 2

CIDR: /30

Subnet mask: 255.255.255.252

Network address: 200.10.20.132

Broadcast address: 200.10.20.135
First Usable address: 200.10.20.133
Last Usable address: 200.10.20.134

2. 172.16.0.0/23

a.VLAN_30 - 250 hosts

Host bits: 8

CIDR: /24

Subnet mask: 255.255.255.0

Network address: 172.16.0.0

Broadcast address: 172.16.0.255

First Usable address: 172.16.0.1

Last Usable address: 172.16.0.254

b.VLAN_10 - 120 hosts

Host bits: 7

CIDR: /25

Subnet mask: 255.255.255.128

Network address: 172.16.1.0

Broadcast address: 172.16.1.127

First Usable address: 172.16.1.1

Last Usable address: 172.16.1.126

c.VLAN_40 - 60 hosts

Host bits: 6

CIDR: /26

Subnet mask: 255.255.255.192

Network address: 172.16.1.128

Broadcast address: 172.16.1.191

First Usable address: 172.16.1.129

Last Usable address: 172.16.1.190

d.VLAN_20 - 30 hosts

Host bits: 5

CIDR: /27

Subnet mask: 255.255.255.224

Network address: 172.16.1.192

Broadcast address: 172.16.1.223

First Usable address: 172.16.1.193

Last Usable address: 172.16.1.222

3.10.5.128.0/20

a.Campus_D - 2000 hosts 
Host bits: 11

CIDR: /21

Subnet mask: 255.255.248.0

Network address: 10.5.128.0

Broadcast address: 10.5.135.255

First Usable address: 10.5.128.1

Last Usable address: 10.5.135.254

b.Campus_A - 1000 hosts

Host bits: 10

CIDR: /22

Subnet mask: 255.255.252.0

Network address: 10.5.136.0

Broadcast address: 10.5.139.255

First Usable address: 10.5.136.1

Last Usable address: 10.5.139.254

c.Campus_C - 500 hosts

Host bits: 9

CIDR: /23

Subnet mask: 255.255.254.0

Network address: 10.5.140.0

Broadcast address: 10.5.141.255

First Usable address: 10.5.140.1

Last Usable address: 10.5.141.254

d.Campus_B - 200 hosts

Host bits: 8

CIDR: /24

Subnet mask: 255.255.255.0

Network address: 10.5.142.0

Broadcast address: 10.5.142.255

First Usable address: 10.5.142.1

Last Usable address: 10.5.142.254

4.10.0.0.0/12

Region_1 — 30,000 hosts

- Host bits: 15

- CIDR: /17

- Mask: 255.255.128.0

- Network: 10.0.0.0

- Broadcast: 10.0.127.255

- First: 10.0.0.1

- Last: 10.0.127.254

Region_3 — 15,000 hosts

- Host bits: 14

- CIDR: /18

- Mask: 255.255.192.0

- Network: 10.0.128.0

- Broadcast: 10.0.191.255

- First: 10.0.128.1

- Last: 10.0.191.254

Region_2 — 8,000 hosts

- Host bits: 13

- CIDR: /19

- Mask: 255.255.224.0

- Network: 10.0.192.0

- Broadcast: 10.0.223.255

- First: 10.0.192.1

- Last: 10.0.223.254

Region_4 — 4,000 hosts

- Host bits: 12

- CIDR: /20

- Mask: 255.255.240.0

- Network: 10.0.224.0

- Broadcast: 10.0.239.255

- First: 10.0.224.1

- Last: 10.0.239.254

P2P_1 — 2 hosts

- Host bits: 2

- CIDR: /30

- Mask: 255.255.255.252

- Network: 10.0.240.0

- Broadcast: 10.0.240.3

- First: 10.0.240.1

- Last: 10.0.240.2

P2P_2 — 2 hosts

- Host bits: 2

- CIDR: /30

- Mask: 255.255.255.252

- Network: 10.0.240.4

- Broadcast: 10.0.240.7

- First: 10.0.240.5

- Last: 10.0.240.6